3Dlabs

Software
Release Note

Windows 95 Display Driver for the 3D Blaster from Creative Labs

Introduction

This note describes the 3Dlabs Windows 95 Display Driver for the 3D Blaster from Creative Labs and includes instructions on the installation procedure. The display driver includes support for DirectDraw and Direct3D. Beta Release

Change History

Release Note 121.04.054

| Issue | Date | Change |
| --- | --- | --- |
| 1 | 2-Oct-95 | Creation |
| 2 | 30-Nov-95 | Updated for Beta 0.9 |
| 3 | 25-Jan-96 | Updated for Beta 0.95 |
| 4 | 1-Mar-96 | Updated for Beta 4.02.0096 |
| 5 | 24-Apr-95 | Updated for Beta 4.02.0098 |

© Copyright 3Dlabs Inc. Ltd.  1996.  All rights reserved world-wide.

The material in this document is the intellectual property of 3Dlabs.  It is provided solely for information.  You may not reproduce this document in whole or in part by any means.  While every care has been taken in the preparation of this document, 3Dlabs accepts no liability for any consequences of its use.  Our products are under continual improvement and we reserve the right to change their specification without notice.

3Dlabs is the worldwide trading name of 3Dlabs Inc. Ltd.
GLINT is a registered trademark of 3D Labs.

PERMEDIA is a trademark of 3D Labs

OpenGL is a trademark of  Silicon Graphics, Inc.  UNIX is a registered trademark of UNIX System Laboratories.  Windows, Windows 95, Win32 and Windows NT are trademarks of Microsoft Corp.  AutoCAD is a trademark of AutoDesk Inc.  MicroStation is a trademark of Intergraph Corp.

3D Blaster is a trademark of Creative Labs.

All other trademarks are acknowledged.

| 3Dlabs Inc. 181 Metro Drive, Suite 520, San Jose, CA 95110 United States Tel: (408) 436 3455 Fax: (408) 436 3458 | 3Dlabs Ltd. Meadlake Place Thorpe Lea Road. Egham Surrey, TW20 8HE United Kingdom Tel: +44 (0) 1784 470555 Fax: +44 (0) 1784 470699 |
| --- | --- |

# Introduction

This document describes 3Dlabs’ Windows 95 Display Driver for the Creative Labs 3DBlaster.  The driver includes support for Direct3D and DirectDraw. For hardware accelerated Direct 3D and Retained Mode Direct3D.

This document should be read in conjunction with the README.TXT file on the installation floppy, which contains details of any enhancements and/or bug fixes that have been made subsequent to these release notes being written.

## Release Identity

Once the driver has been installed and the machine rebooted, the display driver release number can be determined by inspecting the Driver page of the Device Manager property sheets for the installed board. Note you need to click on the glint.drv file to display the version.

## Prerequisites

- Windows 95 (Final Release)

- Intel 486 processor or later

- Creative 3Dblaster (2 + 2MB memory configuration recommended)

# Important Information

This driver is released to gives developers early access to hardware accelerated Direct3D .  There are currently no plans to productise this driver for the 3DBlaster.  The final release of this driver will be productised for 3Dlabs Permedia device.  During the interim this 3DBlaster release will continue to be developed and supported.  Updates of this driver will be placed on the 3D Labs web site (http://www.3dlabs.com).

Installing the 3Dlabs Windows 95 Display Driver for the 3D Blaster, replaces Creative Labs existing display driver and so may affect the use of CGL based applications running under Windows 95.

Technical Support Contacts:

- Email: support@3dlabs.com

- Phone: +44 (0) 1784 476 651

- Fax: +44 (0) 1784 470 699

# Installation

*** IMPORTANT *** If you are installing a Windows 95 3D Blaster driver for the first time, you must carefully follow the instructions supplied with your 3D Blaster. Specifically you should physically remove any 3D Blaster card from your machine before installing Windows 95. You should also ensure that you do not plug in your 3D Blaster card until after you have installed the Windows 95 3D Blaster drivers. This is due to a bug in the Windows95 configuration manager. 3Dlabs are working with Microsoft to resolve this.

- If you have not already installed the 3D Blaster Windows 95 Display driver from Creative Labs, supplied with your card, install this first to enable the workaround for the configuration manager problem.

- Once you have the Creative Labs Windows 95 Display Driver installed and running, you can install the replacement drivers supplied.

1.3 Install the new drivers as follows:

Right click on the "My Computer" ICON, and select "Properties".

Access the Device Manager.

Expand "Display adapters".

Select the existing Creative Labs display driver.

Select “Properties”.

Select the “Driver” tab.

Select “Change Driver”.

In the box  that appears select “Have Disk”.

Choose the directory in which the 3D Blaster drivers reside.

The system will now reboot and use the new Display driver.

- You should now have a working 3D Blaster driver running at 640x480 16 Bit color. Use the Display Control Panel to change to other resolutions.

Note that the Direct3D HAL driver currently only supports 16-Bit modes in this BETA release.

Also note that most of the Direct3D sample applications will not have enough card memory for the hardware back-buffer unless they are started in 640x400 on a 1+1 MB 3D Blaster card. (the applications usually report this as “AddAttachedBuffer failed for Z Buffer” ).

# Resolutions and Color Depths

The following modes are supported by this display driver release.  The 32768 color modes are the only ones supporting Direct3D hardware acceleration.

| Resolution | Colors | Additional |
| --- | --- | --- |
| 640x400 | 256 |  |
| 640x400 | 32768 |  |
| 640x400 | True Color |  |
| 640x480 | 256 |  |
| 640x480 | 32768 | Full screen Hardware Double Buffered needs 2 + 2MB card |
| 640x480 | True Color |  |
| 800x600 | 256 |  |
| 800x600 | 32768 | Hardware Double Buffered needs 2 + 2MB card |
| 800x600 | True Color |  |
| 1024x768 | 256 |  |
| 1024x768 | 32768 | Hardware Double Buffered needs 2 + 2MB card |
| 1152x870 | 256 |  |
| 1152x870 | 32768 | Needs 2 + 2MB card |
| 1280x1024 | 256 | Needs 2 + 2MB card |

All of the above modes are available at both 60Hz and 75Hz monitor refresh rates, except 640x400 which is only available at 60Hz. In addition, the 640x480 resolution is available at 72Hz.

# 3DBlaster Control Applet

3DBlaster specific variables can be directly changed by the 3DBlaster display properties sheet.  This applet is accessible from the device manager on accessing the properties of the 3D Blaster board.  The property sheet allows users to configure specific variables, e.g. screen refresh rate.

# Direct Draw Support

The driver contains support for hardware optimised DirectDraw functionality, which is used by DirectDraw games as well as by the Direct3D Hardware Abstraction Layer.

# Direct3D

The driver contains a Direct3D HAL driver in BETA form.  This enables hardware acceleration of Direct3D applications.  As this is an early BETA, some features are currently still under development and there are limitations which should be understood before attempting to run Direct3D or Reality Lab programs.  The main limitations are listed below.

## Direct3D HAL BETA - Implemented Features.

- Fogging, Z Buffering.

- Gouraud and flat shaded primitives.

- Textures are supported (with or without Perspective Correction).

- Line and point primitives.

- Sub-Pixel Correction.

## Direct3D HAL BETA - Current limitations.

- Alpha blending and  stipple patterns are not supported.

- Specular textures are not supported (these will render as normal textures).

- The display must be in a 16Bit mode to run the Direct3D Demos.

## Known Anomalies and Restrictions

The following errors have been noted during testing of this software release:

- When using a software cursor there may occasionally be corruption when moving “solid” windows around the screen.

- Windows 95 must not be either installed or booted with a 3D Blaster card present  in the system unless the drivers have been previously installed. 3Dlabs are working with Microsoft to fix this problem.

- Task switching away from a full screen Direct3D program can cause the application to hang.

- Textured wireframe lines may not display the correct texels under some circumstances.

# Direct3D Retained mode support

Direct3D retained mode applications will run accelerated with this driver.

# OpenGL support

There is no support for OpenGL in this release of the driver for the current 3D Blaster.  OpenGL support will be included in the Permedia release.