3Dlabs

Software
Release Note

BETA Windows 95 Display Driver for GLINT SX.

Introduction

This note describes the Windows 95 Display Driver for the GLINT 300SX reference board (Montserrat), and how to install it. The display driver includes support for DirectDraw and Direct3D in early BETA form.

Change History

Release Note 121.04.042

| Issue | Date | Change |
| --- | --- | --- |
| 1 | 2-Oct-95 | Creation |
| 2 | 30-Nov-95 | Updated for Beta 0.9 |
| 3 | 25-Jan-96 | Updated for Beta 0.95 |

© Copyright 3Dlabs Inc. Ltd.  1996.  All rights reserved world-wide.

The material in this document is the intellectual property of 3Dlabs.  It is provided solely for information.  You may not reproduce this document in whole or in part by any means.  While every care has been taken in the preparation of this document, 3Dlabs accepts no liability for any consequences of its use.  Our products are under continual improvement and we reserve the right to change their specification without notice.

3Dlabs is the worldwide trading name of 3Dlabs Inc. Ltd.
GLINT is a registered trademark of 3Dlabs.

OpenGL is a trademark of  Silicon Graphics, Inc.  UNIX is a registered trademark of UNIX System Laboratories.  Windows, Windows 95, Win32 and Windows NT are trademarks of Microsoft Corp.  AutoCAD is a trademark of AutoDesk Inc.  MicroStation is a trademark of Intergraph Corp.

All other trademarks are acknowledged.

| 3Dlabs Inc. 2010 North First Street, Suite 403 San Jose, CA 95131 United States Tel: (408) 436 3455 Fax: (408) 436 3458 | 3Dlabs Ltd. Meadlake Place Thorpe Lea Road. Egham Surrey, TW20 8HE United Kingdom Tel: +44 (0) 1784 470555 Fax: +44 (0) 1784 470699 |
| --- | --- |

# Introduction

This document describes the Windows 95 Display Driver for the GLINT 300SX reference board, Montserrat.  The driver includes support for OpenGL, Direct3D and DirectDraw.  Note that for hardware accelerated OpenGL, you will need the Installable Client Driver binaries from 3D Labs.  For hardware accelerated Direct 3D and Reality Lab, you will need Release 4.0 of the DirectX PDK.

This document should be read in conjunction with the README.TXT file on the installation floppy, which contains details of any enhancements and/or bug fixes that have been made subsequent to these release notes being written.

## Release Identity

Once the driver has been installed and the machine rebooted, the display driver release number can be determined by inspecting the Driver page of the Device Manager property sheets for the installed board. Note you need to click on the glint.drv file to display the version.

## Prerequisites

¥	Windows 95 (Final Release)

¥	Intel 486 processor or later.

¥	3Dlabs Montserrat board.

# Installation

*** IMPORTANT *** If you are installing a Windows 95 GLINT driver for the first time, you must carefully follow the instructions below. Specifically you should physically remove any GLINT card from your machine before installing Windows 95. You should also ensure that you do not plug in your GLINT card until after you have installed the Windows 95 GLINT drivers. This is due to a bug in the Windows95 configuration manager. 3Dlabs are working with Microsoft to resolve this.

1.1 If you have not yet installed Windows 95, then before doing so power down your machine and physically remove the GLINT card. Then install Windows 95 using your VGA boot card.

1.2 If you installed Windows 95 with a GLINT card physically present in your system or you plugged in your GLINT card before you installed the drivers, you may need to remove some incorrect registry entries that the configuration manager may have created:

Right click on the "My Computer" ICON, and select "Properties".

Access the Device Manager.

Expand "Other Devices" (if present).

Select "PCI Display Adaptor" (if present).

Click the "Remove" button.

Follow the instructions and allow the machine to reboot.

1.3 Install the new drivers as follows:

Open the control panel, and start the "Add New Hardware" wizard.

When prompted check the box to tell Windows 95 NOT to search for the new hardware and click "Next".

Select the "Display adaptor" class, and "Next".

Click "Have Disk" and follow the instructions to start installing from the GLINT driver floppy.

1.4 Upon completion the machine will offer to shutdown to enable you to install the new hardware. Accept the shutdown request, and when you have powered down, physically install the Montserrat board.

1.5 On reboot Windows 95 will recognise the board, select the correct display driver, and will then offer to reboot. Do NOT allow it to do so, but instead:

Select "Continue".

Right click on the "My Computer" ICON, and select "Properties".

Access the Device Manager.

Expand "Display adapters".

Select your existing VGA compatible display adapter.

Un-check the "Original Configuration (Current)" box.

Allow the machine to reboot when prompted.

1.6 Check the Device Manager to ensure the GLINT drivers are being used. The icon for the VGA card should contain a monitor which is crossed out where the GLINT driver’s icon should be just a monitor. If the GLINT icon is crossed out, then there were additional problems with the installation. This can happen if you plugged in the GLINT board before you installed the drivers. Check the properties for the GLINT board. If there is a resource conflict with the VGA driver, you may need to remove the VGA driver from the Device Manager and reboot.

1.7 You should now have a working GLINT driver running at 640x480 256 colors. Use the Display Control Panel to change to other resolutions.  Note that the Direct3D HAL driver will only work in 16-Bit modes in this BETA release.

# Resolutions and Color Depths

The following modes are supported by this display driver release.  The 32768 color modes are required for Direct3D hardware acceleration.

| Resolution | Colors |
| --- | --- |
| 640x400 | 256 |
| 640x400 | 32768 |
| 640x400 | True Color |
| 640x480 | 256 |
| 640x480 | 32768 |
| 640x480 | True Color |
| 800x600 | 256 |
| 800x600 | 32768 |
| 800x600 | True Color |
| 1024x768 | 256 |
| 1024x768 | 32768 |
| 1024x768 | True Color |
| 1152x870 | 256 |
| 1152x870 | 32768 |
| 1152x870 | True Color |
| 1280x1024 | 256 |
| 1280x1024 | 32768 |

All of the above modes are available at both 60Hz and 75Hz monitor refresh rates, except 640x400 which is only available at 60Hz. In addition, the 640x480 resolution is available at 72Hz.

# Glint Control Applet

GLINT specific variables can be directly changed by the GLINT display properties sheet.  This applet is accessible from the device manager on accessing the properties of the GLINT board.

The ‘GLINT Controls’ property sheet allows the user to change the clock frequency of the GLINT board, and to configure other GLINT specific variables. These other variables should not be changed unless specified by other 3Dlabs releases.

# Registry Entries

Most of the registry configuration can be done from the Glint Control Applet. However, there is one entry which could be useful. It should be set in:

\HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\Class\Display\<DNUM>\DEFAULT

Where <DNUM> is the number Windows 95 has allocated the Glint board. This can be found by selecting all the keys in the Display key in turn. The correct key for the Glint board should have a DriverDesc Entry defined to “3Dlabs Glint 300SX reference board”.

VTGSerialClock

If defined this register is used to program the GLINT VTGSerialClk register. If not defined the default Montserrat value of 6 is used.

This register controls some fundamental VRAM size values. It should usually only be modified by the glint.inf file supplied with the driver for a given board.

# Direct Draw Support

The driver contains support for hardware optimised Direct Draw functionality, which is used by DirectDraw games as well as by the Direct3D HAL.

# Direct3D, Reality Lab support.

The driver contains a Direct3D HAL driver in BETA form.  This enables hardware acceleration of Direct3D and Reality Lab.  Additionally, version 4.0 of the DirectX PDK will be required.  As this is an early BETA, there are limitations which should be understood before attempting to run Direct3D or Reality Lab programs.   The main limitations are listed below.   The HAL driver is currently capable of rendering Gourand and flat shaded primitives, with specular highlights.  The HAL driver also supports Z Buffering.

Bug reports and feedback are welcome and can be mailed to: chrism@3dlabs.com

## Direct3D HAL BETA limitations.

- Release 4.0 of the DirectX PDK supports Direct3D in 16 and 8 bit modes.  The Direct3D HAL driver for GLINT hardware will only run in the 16 Bit mode in this BETA.

- This release of the Direct3D HAL has not been tested on a Revision 1 GLINT 300SX.

- The Direct3D HAL driver does not support texturing when using a GLINT 300SX.  The Direct3D demos will detect this and the texture options will not be available while running the HAL driver.

- Line and Point primitives are not supported in this BETA.

- Fogging and Anti-Aliasing are not supported in this BETA.

# OpenGL support

An Installable Client Driver for OpenGL is required for accelerated OpenGL. This must be obtained from 3D Labs.  The settings below are used in conjunction with the OpenGL Installable Client Driver.

## Additional OpenGL Configuration Options

There are a number of entries within the registry which control the behaviour of OpenGL, in this BETA release:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\glint\Device0

¥	GlintDMA.NumberOfSubBuffers - This is used to determine how the DMA buffer is divided up into separate DMA-able buffers. This can have considerable performance implications, 2 is the most optimum setting in this BETA, assuming a DMA buffer size of 65535..

¥	3DInterfaceBuffer.SizeLongs - This is used to control the size in longs of the shared memory block allocated to each individual rendering context. This buffer is used when DMA is disabled or the entire DMA buffer is in use. The default value is 0x2000.

¥	OpenGL.NoDMA - This can be set to 1 to indicated that you do not wish to use DMA buffers.  This can be helpful if there is a problem with a machine that cannot do DMA.

The environment variable GLINT_DONT_USE_FCP can be used to disable the use of the Fast Clear Planes. This is done by setting the variable to TRUE within the system applet of the control panels.

# OpenGL Supported Configurations

The BETA Installable Client Driver together with the 3Dlabs Glint Display Driver allow the user to boot into a range of modes which are a trade off between  color resolution, screen resolution and the ability to do color space double buffering (which can improve animation performance).

Note that when running at a resolution greater than 800 by 600 in True Color Mode (16 Million Colors) on a board with 4Mbyte of VRAM, double buffered OpenGL applications will revert to using the generic OpenGL implementation.

# Known Anomalies and Restrictions

The following errors have been noted during testing of this software release:

- When using a software cursor there may occasionally be corruption when moving “solid” windows around the screen.

- Windows 95 must not be either installed or booted with a GLINT card present  in the system unless the drivers have been previously installed. 3Dlabs are working with Microsoft to fix this problem.

- When running 3Dlabs OpenGL on a Rev 1 GLINT board the hardware cursor will often become corrupted and occasionally cause the machine to hang.

# Known Anomalies and Restrictions of Direct3D HAL

- Task switching away from a full screen Direct3D program can cause the application to hang.

- Upon running a second Direct3D program, the 3D objects may not render correctly, and the Z Buffering will fail.