This directory contains the Release 2.23 drivers for the NV1 Multimedia Accelerator. These drivers are compatible with most boards boards based on the NV1 chip from NVidia Corporation. If you have any problems using these drivers, please contact your board manufacturer.

Adding the NV1 Board to a Windows 95 System

This procedure assumes you are currently running Windows 95 on another video card. Display drivers and multimedia drivers are installed manually and must be added in the correct order: display drivers first, followed by multimedia drivers.

1. Install the NV1 board.

2. Install the Display Adapter driver.

A. Power on your system. Windows 95 displays a dialog box indicating

"New Hardware Found—A PCI VGA Compatible Display Adapter."

B. Choose to install drivers from a disk provided by the hardware manufacturer.

C. Type the appropriate path to the correct directory, and choose OK.

Drivers are copied to the WINDOWS\SYSTEM directory.

D. Choose No when asked to restart the system.

3. Install the Multimedia Device Driver.

A. Windows 95 displays a dialog box indicating "New Hardware Found—A PCI Multimedia Device." Choose to

install drivers from a disk provided by the hardware manufacturer.

B. Type the appropriate path to the correct directory, and choose OK.

Drivers are copied to the WINDOWS\SYSTEM directory.

4. Update the NVIDIA MPU401 driver for General MIDI in DOS shell.

A. From Control Panel, choose Add New Hardware.

B. The Add New Hardware Wizard page appears. Choose Next.

C. You are asked if you want Windows to search for your new hardware. Select No, and choose Next.

D. Choose Sound, Video and Game Controllers, and then choose Next.

E. A list of manufacturers and models appears. Select NVIDIA as the manufacturer and the NVIDIA MPU401

Driver for General MIDI in DOS Shell as the model. Choose the Have Disk button.

F. Type the appropriate path to the correct directory, and choose OK:

G. Choose OK to approve NVIDIA MPU401 Driver for General MIDI in DOS Shell.

The MPU401 driver needs an I/O address and may be unable to use the default address of 230. If a 	      	     	      	      Resource Type page appears, the 230 address is already in use by another device in your system. An 	                        	      	      alternate range appears on the Resource Type screen. Take note of this I/O port address so you can

configure your games as necessary.

H. Choose OK to approve the NVIDIA MPU401 Multimedia Device Properties page.

Drivers are copied to the WINDOWS\SYSTEM directory.

I. Choose Next, and choose Finish to end the procedure.

J. If you want to install the NV1 joystick port, choose No when asked to restart the system.

5. Install the Microsoft joystick driver.

A. Open Control Panel.

(Start, Settings, Control Panel)

If you see a Joystick icon in this window, the NV1 joystick port is correctly set up for Windows 95 use; 	     	     	     however, you may reconfigure it if necessary (see"Configuring Joystick" on page 36). Otherwise, choose

Add New Hardware and choose Next.

B. You are asked if you want Windows to search for your new hardware. Select No, and choose Next.

C. A list of hardware types appears. Select Sound, Video, and Game Controllers, and choose Next.

D. A list of manufacturers appears. Select Microsoft.

E. A list of models appears. Select Gameport Joystick, and choose Next.

F. Choose Next on the properties screen.

G. If prompted, type the path to the Windows 95 CD-ROM, and choose OK.

H. Choose Yes when asked to restart the system.

Updating NV1 Drivers

This procedure assumes you are running NV1 and Windows 95 and want to update your NV1 drivers to the current version. Display drivers, multimedia drivers, and MPU-401 drivers are updated individually. The MIDI bank file setting should be confirmed.

1. Update the Display Adapter driver.

A. Open Device Manager.

(Start, Settings, Control Panel, System)

B. Choose Display Adapters.

C. Choose NVIDIA NV1 VGA-Compatible device.

D. Select the Driver tab, and choose the Change Driver button.

E. Choose the Have Disk button.

F. Type the appropriate path to the updated drivers.

G. Choose OK to approve Select Device page.

H. Choose OK to approve the NVIDIA NV1 VGA-Compatible Adapter Properties page.

Drivers are copied to the WINDOWS\SYSTEM directory.

I. Windows asks you to insert the Windows 95 NV1 Driver Disk. Click OK, retype the same path as in step F, and click 	    OK.

J. Choose No when asked to restart the system.

2. Update the Multimedia Device driver.

A. From Device Manager, Choose Sound, Video and Game Controllers.

B. Choose NVIDIA NV1 PCI Multimedia Device.

C. Select the Driver tab, and choose the Change Driver button.

D. Choose the Have Disk button.

E.  Type the appropriate path to the updated drivers.

F. Choose Next to approve Select Device page.

G. Choose OK to approve the PCI Multimedia Device Properties page.

Drivers are copied to the WINDOWS\SYSTEM directory.

H. Windows may ask you to insert the Windows 95 NV1 Driver Disk. Choose OK, retype the same path as in step E.

I. Choose No when asked to restart the system.

3. Update the NVIDIA MPU401 driver for General MIDI in DOS shell.

A.  From Device Manager, Choose Sound, Video and Game Controllers.

B. Choose NVIDIA MPU-401 Driver for General Midi in a DOS shell.

C. Select the Driver tab, and choose the Change Driver button.

D. Choose the Have Disk button.

E.  Type the appropriate path to the updated drivers.

F. Choose Next to approve Select Device page.

G. Choose OK to approve NVIDIA MPU401 Driver for General MIDI in DOS Shell.

The MPU401 driver needs an I/O address and may be unable to use the default address of 230. If a Resource Type

page appears, the 230 address is already in use by another device in your system. An alternate range appears on the

Resource Type screen.

Take note of this I/O port address so you can configure your games as necessary.

H. Choose OK to approve the NVIDIA MPU401 Multimedia Device Properties page.

Drivers are copied to the WINDOWS\SYSTEM directory.

I. Choose Yes when asked to restart the system.