3Dlabs

Software
Release Note

BETA Windows 95 Display Driver/DirectX Driver for PERMEDIA & GLINT 500 TX

Introduction

This note describes the Windows 95 Display Driver for the PERMEDIA Reference board and the GLINT 500TX reference board, and how to install it. The display driver includes support for DirectDraw and Direct3D.

Change History

Release Note 121.04.054

| Issue | Date | Change |
| --- | --- | --- |
| 1 | 2-Oct-95 | Creation |
| 2 | 30-Nov-95 | Updated for Beta 0.9 |
| 3 | 25-Jan-96 | Updated for Beta 0.95 |
| 4 | 1-Mar-96 | Updated for Beta 4.02.0096 |
| 5 | 24-Apr-96 | Updated for Beta 4.02.0098 |
| 6 | 09-July-96 | Updated for Beta 4.02.0101 |
| 7 | 10-Sept-96 | Updated for Beta 4.03.00.2001 |

© Copyright 3Dlabs Inc. Ltd.  1996.  All rights reserved world-wide.

The material in this document is the intellectual property of 3Dlabs.  It is provided solely for information.  You may not reproduce this document in whole or in part by any means.  While every care has been taken in the preparation of this document, 3Dlabs accepts no liability for any consequences of its use.  Our products are under continual improvement and we reserve the right to change their specification without notice.

3Dlabs is the world-wide trading name of 3Dlabs Inc. Ltd.
GLINT is a registered trademark of 3Dlabs.

PERMEDIA is a trademark of 3D Labs

OpenGL is a trademark of  Silicon Graphics, Inc.  UNIX is a registered trademark of UNIX System Laboratories.  Windows, Windows 95, Win32 and Windows NT are trademarks of Microsoft Corp.  AutoCAD is a trademark of AutoDesk Inc.  MicroStation is a trademark of Intergraph Corp.

All other trademarks are acknowledged.

| 3Dlabs Inc. 181 Metro Drive, Suite 520, San Jose, CA 95110 United States Tel: (408) 436 3455 Fax: (408) 436 3458 | 3Dlabs Ltd. Meadlake Place Thorpe Lea Road. Egham Surrey, TW20 8HE United Kingdom Tel: +44 (0) 1784 470555 Fax: +44 (0) 1784 470699 |
| --- | --- |

# Introduction

This document describes the Windows 95 Display Driver for the PERMEDIA and GLINT 500TX reference boards.  The driver includes support for Direct3D and DirectDraw.  Note that for hardware accelerated OpenGL, the Installable Client Driver binaries available from 3Dlabs are needed.

## Release Identity

Once the driver has been installed and the machine rebooted, the display driver release number can be determined by inspecting the Driver page of the Device Manager property sheets for the installed board. Note you need to click on the glint.drv file to display the version.

## Prerequisites

¥	Windows 95 (Final Release)

¥	Intel 486 processor or later.

¥	PERMEDIA or 500TX reference board.

# Important Information

This driver provides support for PERMEDIA and GLINT 500TX and is compatible with DirectX 1, 2 and 3.  Further releases of this driver can also be obtained from the  3D Labs web site (http://www.3dlabs.com).

Technical Support Contacts:

- Email: support@3dlabs.com

- Phone: +44 (0) 1784 476 651

- Fax: +44 (0) 1784 470 699

# Installation

Fitting a new card:

- Please refer to the original instructions provided with your PC for information on how to replace the graphics board. Remember to use anti-static precautions at all times to protect the hardware from static electricity.  If in any doubt about the installation consult a qualified PC engineer.

- Once your original graphics board has been removed, replace it with the 3D Labs card.  Ensure that it is firmly seated and secured to the machine using the appropriate fastenings.  Note that the PERMEDIA and GLINT 500TX boards are VGA enabled, and there should be no other VGA in the machine once the card has been fitted.  If your PC motherboard has built-in VGA it should be disabled.  Again refer to the original equipment manufacturer’s manuals for details on how to do this.

- Fit the PC casing, switch on and boot Windows 95.

- After the login procedure, your new card should be detected and Windows will ask for the correct drivers.  Insert the floppy disk containing the 3D Labs drivers and install them.  Upon completion, Windows will reboot using the new card.  You should start in at least a 16Bit mode for Direct3D acceleration.

Updating existing drivers:

- Whilst running Windows95, open the control panel and choose “Display”.

- Navigate to the “Settings” tab at the top of the panel, and choose “Change Display Type” or “Advanced Properties”.   From here you can change the driver.  Select “Change” and “Have Disk”.  Windows will now prompt for the installation floppy.  Follow the usual procedure to install the drivers.

- Windows will now reboot your machine using the new drivers.

Updating your monitor type:

The 3Dlabs drivers will try to use the optimal refresh rate that the monitor can handle.  For this reason it is important to correctly set up the monitor type to match your display.  If the monitor is over-specified in the Windows95 control panel settings, it is possible that the drivers will attempt to use a refresh rate that is beyond the capabilities of your monitor.  To correct this please ensure that you have correctly specified your monitor type.  The monitor settings can be found in the “Display” settings in the control panel.

Replacing older drivers:

The above procedures should work correctly for the 3Dlabs reference cards.  If you experience problems with installing the drivers, and you have previously installed 3Dlabs drivers on the machine, there may be a conflict.  Follow the steps below to resolve it:

- Boot Windows 95 into safe mode.  This can be done by pressing “F8” just after the machine has rebooted, when the message “Starting Windows 95…” appears.

- Open the device manager.  To do this right-click on the “My computer” icon and choose “Properties”.  Once the device manager is open, select the “Device Manager” tab at the top of the property sheet.

- Look at the list of devices on your machine.  If there are any listed under “3D Accelerators”, such as “Delta Geometry Processor” or “GLINT 500TX”, remove them.  Next look at the “Display Adapters” section.  If any 3D Labs devices, e.g. “PERMEDIA” or “GLINT Compatible VGA”, are listed remove them.

- Close the device manager.

- Open explorer and navigate to the “Windows” directory.  In here you will find a directory called “Inf”.  Look inside this directory for file names that begin with “oem”.  Examine each of the OEM files in turn by double clicking to load them into Notepad.  If any of the files contain references to 3Dlabs, PERMEDIA, or GLINT, delete them.

- Windows is now clean of all references to 3D Labs hardware.  Reboot the machine normally and auto-detection of the hardware will begin again.  Remember to supply the correct driver disk when it is requested.

# Resolutions and Color Depths

The following modes are supported by this display driver release.  This driver does not accelerate Direct3D in 256 color modes, although it does accelerate DirectDraw in these modes.  For hardware acceleration, please ensure you are running in a 16Bit or  32Bit display mode.  If a Direct3D sample application is started in 8Bit mode, it will attempt to change color depth to 16Bits to work with the card.  Note that Direct3D rendering is not supported at resolutions greater than 1280x1024.

The resolutions supported for Windows display are shown below.  Note that the DirectDraw driver will support resolutions different to the Windows desktop (i.e. 512 x 384 on GLINT 500TX and PERMEDIA, 320 x 240, 320 x 200 on PERMEDIA). The memory requirements will differ depending upon the applications you run.  If the Direct3D/DirectDraw applications complain that there is not enough memory, try reducing screen resolution.

PERMEDIA / GLINT 300SX / GLINT 500TX:

| Resolution | Colors |
| --- | --- |
| 640x400 | 256, 32768, True Color |
| 640x480 | 256, 32768, True Color |
| 800x600 | 256, 32768, True Color |
| 1024x768 | 256, 32768, True Color |
| 1280x1024 | 256, 32768, True Color |
| 1600x1200 | 256,32768 |

GLINT 300SX / GLINT 500TX:

| 1152x870 | 256, 32768, True Color |
| --- | --- |
| 1280x1024 | 256, 32768, True Color |
| 1600x1200 | 256, 32768, True Color |

# Direct Draw Support

The driver contains support for hardware optimized DirectDraw functionality, which is used by DirectDraw games as well as by the Direct3D HAL.  The driver is compatible with DirectX 1, 2, and 3.  Please obtain a release of this software if you require DirectDraw / Direct3D acceleration.

# Direct3D

The driver contains a Direct3D HAL driver.  This enables hardware acceleration of Direct3D applications and is compatible with DirectX 2 and DirectX 3.  Some features are not available in this release and these are documented below:

## Direct3D HAL - Current limitations.

- Spans and Single Points are not supported in this release of the drivers.

- Alpha Tests through Direct3D are not supported in this release.  Note that Chroma tests are supported.

- YUV texturing is not supported in this release.

## Known Anomalies and Restrictions

The following errors have been noted during testing of this software release:

- When using a software cursor there may occasionally be corruption, when moving over windows on the screen.   If you experience problems the workaround is to use a hardware mouse cursor - this is set up by changing the pointer type to “Windows Default”.

# New Driver Features

# This release of the Windows95 DirectX driver has several new features to accelerate various aspects of DirectDraw and Direct3D, particularly in the PERMEDIA case.

New PERMEDIA Features:

320 x 240, 320 x 200 Resolutions:

Ultra-Low resolution formats for legacy games originating on console based systems.

Video Acceleration:

The PERMEDIA driver can now take a YUV format DirectDraw surface and blit it to the desktop whilst doing the color conversion to RGB.  This gives high quality, fast decoding of  the YUV format data that is generated by MPEG movie decoders.

Stretch Blits / Mirror Blits:

Stretching is now enabled for all blitting operations and can be combined with mirroring.

ColorKey Blits / Tests:

The driver can now apply ColorKey tests during DirectDraw blits and can also apply ColorKey tests to textured surfaces.  This greatly accelerates applications that require ColorKey blits using DirectDraw surfaces and allows transparent texture effects.

Texture Compression:

PERMEDIA supports a proprietary texture compression scheme when using the Direct3D ALLOC_ON_LOAD mechanism.  The scheme, known as ‘Patching’, greatly accelerates textured rendering with no loss in quality.  Three different modes of operation are supported in the Direct3D driver - see below in “Control Panel Applets”.

System Memory -> Video Memory Blits:

DirectX II onwards allows acceleration of Blits from System memory to video memory.  This is now accelerated by the PERMEDIA driver for standard Blits.

New PERMEDIA & GLINT 500TX Features:

512 x 384 Resolution:

Many games prefer this mode with its good quality/resolution trade-off.  It also saves on valuable card memory.

Alpha Blending / Stippled Alpha Blending:

Both drivers have support for the two methods of blending supported in Direct3D.  Alpha Blending gives true-transparency effects and Stippled Alpha Blending can be used for screen door transparency and shadow effects.

Mono Color Model support:

Both drivers now support the mono color model as well as the RGB model.  This mode can be used to light textures using monochromatic lights.  Note that this will give a slight speed improvement on boards without a Delta chip.

Z Visibility Testing:

Both drivers now provide Z Visibility tests.  This allows objects with many thousands of polygons to be quickly tested for visibility.  Note that retained mode will automatically use this feature to accelerate rendering.

# GLINT Control Applets

PERMEDIA/GLINT specific variables can be changed in two places.  The hardware specific settings such as Interrupts, etc. can be changed from the device manager in the ‘GLINT Controls’ property sheet.  The available settings are:

Max Refresh: 	Controls the maximum refresh rate that the drivers will allow.

DMA Buffers:	Controls the number of DMA buffers that the drivers will allocate.  Setting 0 here 	disables DMA.

DMA Buffer Size:	Controls the size of each DMA buffer.

Font Cache:	Controls the amount of memory allocated for the font cache.  Setting 0 here disables 	font cacheing.

Interrupt Enable:	Enables interrupts for DMA operations.

Offscreen Bitmaps:	Enables the use of Offscreen bitmaps for faster display driver performance.

Intercept GDI:	Intercepts GDI calls for faster GDI graphics operations.

User specific settings for Direct3D are controlled on the desktop display settings panel.  This can be found by right-clicking on the desktop and choosing “Properties”.  Note that not all of these settings are available while running on a 500TX based board.

Selective Patching:	This scheme will patch textures if they are loaded onto the card using the  	Direct3D ALLOC_ON_LOAD scheme.

Always Patch:	This scheme will always patch textures, whether or not they are stored using the 	ALLOC_ON_LOAD scheme.  Note that this can potentially break applications 	that need to render to texture surfaces without compression.

Never Patch:	Never patches textures.

8Bit RGB Textures:	Direct3D Retained mode chooses the texture format that best matches the original 	image file.  This can cause applications to use 8Bit RGB textures when a higher 	resolution texture format would be better.  For this reason 8Bit RGB textures are 	disabled by default.  This check box enables 8Bit RGB textures.

Disable Alpha Textures:	Disables Alpha Texture operations in the Direct3D HAL driver.

True Color Textures:	Enables the use of True Color textures in the Direct3D HAL driver.  This option 	will use a lot of video memory if it is picked by a Direct3D application.

Don’t Force CopyMode:	Allows the Direct3D driver to use texture modulation.  When turned off it allows 	the original color of the texture map to be seen.

Disable Direct3D HAL:	Disables hardware 3D acceleration.

No Vblank Wait:	Stops the DirectDraw HAL driver from waiting for Video Blanking signals before 	it page flips.  This stops full screen applications waiting for the Video Blanking 	interval and allows them to run at full speed.

Filter Stretching:	Filters all stretched blit operations.  This should be off by default as it will slow 		stretch blit performance.  Useful for real time anti-aliasing demos that wish to 	stretch large images down with filtering.